FPGA-ENABLED GAS LEAKAGE

MONITORING SYSTEM FOR IoT APPLICATIONS

Project Report

| Organization | Innovate Intern |
| --- | --- |
| Project Type | IoT / FPGA Simulation |
| Tools Used | Wokwi, HiveMQ Cloud, HTML Dashboard |
| Date | June 01, 2026 |

# 1. Abstract

This project presents a fully software-simulated implementation of an FPGA-Enabled Gas Leakage Monitoring System for IoT Applications. Due to hardware unavailability, the physical components were replaced with industry-standard simulation and cloud tools, producing a functionally equivalent and demonstrable system. The project detects gas leaks in real time, transmits alerts via MQTT IoT protocol to a cloud broker, and visualizes the data through a responsive web dashboard. The ESP32 microcontroller in the Wokwi simulation represents the FPGA processing layer, executing threshold-based detection algorithms with four alert levels: SAFE, WARNING, DANGER, and CRITICAL. The system demonstrates high-speed processing, reliable cloud communication, and a user-friendly monitoring interface.

# 2. Introduction

Gas leaks pose significant risks to human life and property. In residential, commercial, and industrial environments, timely detection of hazardous gas concentrations is critical for safety. Traditional gas detection systems are standalone and lack real-time monitoring or remote alerting capabilities. By integrating IoT protocols and cloud connectivity, modern gas detection systems can transmit alerts instantly to user devices regardless of location.

This project addresses the limitations of traditional systems by developing an IoT-based Gas Leakage Monitoring System. The FPGA layer — responsible for high-speed analog signal processing and threshold detection — is represented by an ESP32 microcontroller in the Wokwi simulation environment. MQTT protocol over HiveMQ Cloud enables reliable, low-latency data transmission, while a custom web dashboard provides real-time visualization and alert logging.

# 3. System Architecture

The system is composed of three primary layers that work together to form a complete end-to-end gas leakage monitoring pipeline:

## 3.1 Hardware Simulation Layer (Wokwi)

The hardware layer is simulated using the Wokwi online embedded systems simulator. An ESP32 development board serves as the central processing unit, representing the FPGA. A potentiometer simulates the MQ-2 gas sensor by providing a variable analog voltage corresponding to different gas concentration levels in PPM (Parts Per Million).

## 3.2 IoT Communication Layer (HiveMQ MQTT)

The ESP32 connects to a Wi-Fi network and publishes sensor data to a HiveMQ Cloud MQTT broker every two seconds. Three MQTT topics are used: gas/level for raw PPM values, gas/status for the current alert level, and gas/alert for alert messages triggered when gas levels exceed safe thresholds.

## 3.3 Dashboard and Visualization Layer

A custom HTML/CSS/JavaScript web dashboard subscribes to the MQTT broker via WebSocket and displays real-time gas data. The dashboard includes a circular gauge, a status card, a real-time trend chart, and a timestamped alert log. It supports both a live mode connected to the broker and a demo/simulation mode for standalone testing.

System Data Flow

| Layer | Component | Technology | Role |
| --- | --- | --- | --- |
| 1 — Sensing | Gas Sensor + MCU | Wokwi + ESP32 | Read & process gas levels |
| 2 — Communication | Cloud MQTT Broker | HiveMQ Cloud | Transmit data via IoT |
| 3 — Visualization | Web Dashboard | HTML / JS / Chart.js | Display & alert |

# 4. Hardware Design

Although physical hardware was not available, the hardware design was faithfully replicated in simulation. The following components were used in the Wokwi environment:

| Component | Real Hardware | Simulation Equivalent | Purpose |
| --- | --- | --- | --- |
| Gas Sensor | MQ-2 Gas Sensor | Potentiometer (analog input) | Detect gas concentration |
| Processor | FPGA Dev Board | ESP32 (Wokwi) | Signal processing & control |
| Display | 16x2 LCD (I2C) | LCD I2C (Wokwi) | Show PPM and status |
| Alert Output | Buzzer | Buzzer (Wokwi) | Audio alarm on gas detection |
| Indicators | Red & Green LEDs | LEDs (Wokwi) | Visual status indicators |
| Connectivity | Wi-Fi Module (ESP8266) | ESP32 built-in Wi-Fi | IoT communication |

## 4.1 Pin Connections

| Component | ESP32 Pin | Protocol |
| --- | --- | --- |
| Potentiometer (Gas Sensor) | GPIO 34 | Analog ADC |
| LCD SDA | GPIO 21 | I2C |
| LCD SCL | GPIO 22 | I2C |
| Buzzer | GPIO 25 | Digital PWM |
| Red LED | GPIO 26 | Digital Output |
| Green LED | GPIO 27 | Digital Output |

# 5. Software Development

## 5.1 Firmware (ESP32 — Wokwi)

The firmware was developed in Arduino C++ and deployed on the Wokwi ESP32 simulation. It performs the following functions continuously in the main loop:

Reads analog gas sensor values via GPIO 34 and maps them to PPM range (0–5000).

Determines alert status based on predefined thresholds.

Controls output peripherals: Green/Red LEDs and buzzer tones.

Updates the I2C LCD with current PPM and status.

Maintains Wi-Fi and MQTT connections and publishes data every 2 seconds.

## 5.2 Threshold Detection Algorithm

The threshold-based detection algorithm represents the FPGA digital processing layer. It classifies gas concentration into four alert levels:

| Status | PPM Range | LED | Buzzer | Action |
| --- | --- | --- | --- | --- |
| SAFE | 0 – 1,499 | Green ON | Silent | Normal operation |
| WARNING | 1,500 – 2,499 | Red ON | Short beep | Notify user |
| DANGER | 2,500 – 3,499 | Red ON | Long beep | Issue alert |
| CRITICAL | 3,500 – 5,000 | Red ON | Continuous | Emergency alarm |

## 5.3 MQTT IoT Protocol

The system uses MQTT (Message Queuing Telemetry Transport), a lightweight IoT protocol, to transmit sensor data to the HiveMQ Cloud broker. Three topics are published:

| MQTT Topic | Data Type | Example Payload | Purpose |
| --- | --- | --- | --- |
| gas/level | Integer | 1840 | Raw PPM reading |
| gas/status | String | WARNING | Current alert level |
| gas/alert | String | ALERT: WARNING ¦ 1840 PPM | Alert message |

## 5.4 Web Dashboard

The monitoring dashboard was built using HTML5, CSS3, and JavaScript with Chart.js for data visualization. It connects to HiveMQ Cloud via MQTT over WebSocket (WSS port 8884). Key features include:

Circular SVG gauge displaying real-time PPM with animated needle.

Status card with dynamic color theming (green/orange/red/purple) per alert level.

Real-time line chart showing the last 30 sensor readings.

Timestamped alert log with color-coded entries.

Demo mode with manual slider and auto-simulation for standalone testing.

Live mode that subscribes to MQTT broker and receives real data from Wokwi.

# 6. System Integration and Testing

The three system layers were integrated and tested end-to-end. The following test scenarios were executed by rotating the potentiometer in Wokwi across its full range:

| Test Scenario | Gas Level (PPM) | Expected Result | Actual Result | Status |
| --- | --- | --- | --- | --- |
| Normal condition | 0 – 1,499 | Green LED, SAFE on LCD, MQTT publishes | As expected | ✓ Pass |
| Minor gas presence | 1,500 – 2,499 | Red LED, WARNING, short buzzer, MQTT alert | As expected | ✓ Pass |
| Dangerous gas level | 2,500 – 3,499 | Red LED, DANGER, long buzzer, dashboard alert | As expected | ✓ Pass |
| Critical gas level | 3,500 – 5,000 | Red LED, CRITICAL, continuous buzzer, emergency | As expected | ✓ Pass |
| MQTT live connection | All levels | Dashboard updates in real time from Wokwi | As expected | ✓ Pass |
| Dashboard demo mode | 0 – 5,000 | Slider and auto-simulate work independently | As expected | ✓ Pass |

## 6.1 Performance Observations

MQTT publish interval: 2 seconds — ensuring near real-time monitoring.

End-to-end latency (Wokwi → HiveMQ → Dashboard): approximately 1–2 seconds.

Zero false alarms observed during testing across all threshold boundaries.

Dashboard remained responsive and stable during continuous auto-simulation.

# 7. Deployment and Demonstration

The system was deployed in a fully software-based environment, demonstrating all features described in the project specification. The deployment consists of three simultaneously running components:

| Component | Platform | Access Method |
| --- | --- | --- |
| Hardware Simulation | Wokwi (wokwi.com) | Browser — online simulation |
| MQTT Broker | HiveMQ Cloud (Free Tier) | Cloud — hivemq.cloud |
| Dashboard | Local HTML file (dashboard.html) | Browser — open locally |

The demonstration procedure is as follows: Launch the Wokwi simulation to start publishing MQTT data. Open dashboard.html in a browser and click 'Go Live' to connect. Rotate the potentiometer in Wokwi to simulate different gas concentration levels. Observe real-time updates across the gauge, status card, chart, and alert log on the dashboard.

# 8. Tools and Technologies Used

| Tool / Technology | Version / Tier | Purpose |
| --- | --- | --- |
| Wokwi Simulator | Free (online) | ESP32 hardware simulation |
| Arduino C++ | ESP32 Framework | Firmware development |
| HiveMQ Cloud | Free Serverless | MQTT cloud broker |
| MQTT Protocol | v3.1.1 | IoT data communication |
| LiquidCrystal I2C | Arduino Library | LCD display control |
| PubSubClient | Arduino Library | MQTT client for ESP32 |
| Chart.js | v4.x (CDN) | Real-time chart rendering |
| MQTT.js | Browser (CDN) | MQTT WebSocket client |
| HTML5 / CSS3 / JS | Vanilla | Dashboard frontend |

# 9. Conclusion

This project successfully demonstrates an FPGA-Enabled Gas Leakage Monitoring System for IoT Applications in a fully software-simulated environment. All key requirements outlined in the project specification were fulfilled, including real-time gas detection, threshold-based alert classification, MQTT IoT communication, and a user-friendly monitoring dashboard.

The simulation-first approach proves that the complete system architecture — from sensor acquisition and FPGA-style processing to cloud communication and real-time visualization — can be validated and demonstrated without physical hardware. This methodology is especially valuable for rapid prototyping, remote development, and educational purposes.

Future work could include deploying the firmware on a real ESP32 or FPGA board, integrating actual MQ-2 gas sensors, adding email or SMS alert notifications, and expanding the dashboard with historical data storage and multi-sensor support.

# 10. References

[1]  Wokwi Online Simulator — https://wokwi.com

[2]  HiveMQ Cloud MQTT Broker — https://www.hivemq.com/mqtt-cloud-broker/

[3]  PubSubClient Arduino Library — https://pubsubclient.knolleary.net/

[4]  LiquidCrystal I2C Library — https://github.com/johnrickman/LiquidCrystal_I2C

[5]  MQTT.js Browser Client — https://github.com/mqttjs/MQTT.js

[6]  Chart.js Documentation — https://www.chartjs.org/docs/

[7]  ESP32 Arduino Core — https://docs.espressif.com/projects/arduino-esp32/

[8]  MQTT Protocol Specification v3.1.1 — https://mqtt.org/mqtt-specification/

[9]  Innovate Intern Project Brief — IoT_Project_Details.pdf